Project Report: Power Consumption Prediction (Zone 3)

1. Executive Summary

The primary objective of this project was to construct a robust machine learning model capable of accurately predicting power consumption in Zone 3. By analyzing historical weather conditions, emission diffusion metrics, and historical consumption data, we developed a highly accurate forecasting tool. The final deployed model—a Random Forest Regressor—achieved an $R^2$ score of 0.846, successfully explaining approximately 85% of the variance in Zone 3's power demand on unseen future data.

2. Research Methodology & Data Processing

The dataset provided consisted of 10-minute interval recordings spanning a single year. A major constraint of single-year datasets is the risk of the model memorizing a specific timeline rather than learning true underlying behaviors. To ensure the model remains generalizable for future forecasting, we executed the following preprocessing steps:

- Temporal & Seasonal Feature Extraction: We dropped the absolute Datetime index and extracted cyclical, structural features: Month, DayOfWeek, and Hour. This forces the model to learn daily and annual seasonal patterns.

- Autoregressive Lag Features: Because power consumption is highly autocorrelated (current demand is strongly linked to past demand), we engineered two critical historical features:

  - Lag_1h: Power consumption exactly 1 hour prior.

  - Lag_24h: Power consumption exactly 24 hours prior.

- Chronological Validation: To simulate real-world forecasting and prevent data leakage, we avoided standard random train/test splitting. The first 80% of the timeline was used for training, while the final 20% was held out as strictly unseen test data.

3. Exploratory Data Analysis (EDA) & Key Insights

Before modeling, we conducted a visual exploration of the dataset to uncover underlying patterns and feature correlations. The key insights are as follows:

- Correlation Analysis: Temperature is the strongest environmental driver of power consumption in Zone 3 (positive correlation of ~0.49). Conversely, Humidity exhibits a slight negative correlation.

- Daily Seasonality (Hourly Trend): Power consumption reliably hits its lowest point during the early morning hours (approx. 2:00 AM – 5:00 AM) and ramps up to a distinct peak window in the evening (approx. 6:00 PM – 9:00 PM).

- Annual Seasonality (Monthly Distribution): Observing the spread of data across months revealed clear structural shifts in base-load consumption throughout the year, confirming the necessity of including the Month feature to adjust baseline predictions accurately across seasons.

- Non-Linear Relationships: The scatter plot mapping Temperature against Zone 3 Power Consumption demonstrates a non-linear relationship. Extreme temperatures cause exponential, rather than proportional, spikes in energy demand.

4. Model Architecture & Performance

Chosen Model: Random Forest Regressor

Justification: We selected a Random Forest algorithm over linear regression models because power consumption exhibits highly non-linear relationships with weather (e.g., the U-shaped curve of heating and cooling demand). Random Forest natively captures these non-linearities and complex interactions (such as the combined effect of Time of Day and Temperature) without overfitting.

Evaluation Metrics (on unseen test data):

- Mean Absolute Error (MAE): 1,019.94 kWh

- Root Mean Squared Error (RMSE): 1,293.36 kWh

- R-Squared ($R^2$): 0.8466 (84.66%)

Feature Importance:

The model revealed that the most dominant predictors for Zone 3's power consumption are Lag_24h (yesterday's demand at the exact same time), followed by Lag_1h, and Hour of the day. Among the environmental metrics, GeneralDiffuseFlows and Temperature had the highest influence on grid strain.

5. Actionable Recommendations & Strategic Insights

Based on our research findings and the model's behavior, we recommend the following strategic actions to optimize resource allocation and promote efficiency in Zone 3:

- Implement Day-Ahead Load Shifting: Because the Lag_24h feature is the strongest predictor of current demand, dispatch operators can confidently use the previous day's load curve as a foundational baseline. We recommend introducing "Time-of-Use" pricing to incentivize consumers to shift heavy appliance usage away from these highly predictable evening peak windows, thereby reducing overall grid strain.

- Weather-Indexed Peaker Plant Readiness: With Temperature showing a strong correlation to demand spikes, grid operations should link local meteorological forecasts directly to the standby procedures of supplementary power sources. Anticipating these temperature-driven peaks will reduce reliance on expensive, last-minute spot-market energy purchases.

- Sustainability via Battery Storage Integration: Given that the daily cyclical nature (Hour) heavily drives the peaks and valleys in consumption, Zone 3 is an ideal candidate for utility-scale battery storage. Excess renewable energy generated during off-peak hours (e.g., late night/early morning) can be stored and discharged during the predictable mid-day to evening surges. This will cut reliance on high-emission peak-hour fossil fuels and support sustainable expansion.

6. Deliverables Provided

- Jupyter Notebook: Zone3_PowerPrediction.ipynb (Contains full EDA, feature engineering, and model training code).

- Visualizations: EDA graphs and actual vs. predicted performance charts.

- Final Report: This document detailing methodology, architecture justification, and strategic recommendations.